Haply Inverse3 + UFactory 850

Haptic Teleoperation Demo  —  Quick Reference

WHAT IS THIS?

You are controlling a real robotic arm using the Haply Inverse3 haptic handle. Your hand movements are mirrored by the robot in real time. The force-torque sensor on the robot sends contact forces back to the handle — so you can feel what the robot feels.

| Input Device - Haply Inverse3 haptic handle - VerseGrip Wireless Stylus - 6 degrees of freedom input | Robot System - UFactory 850 robotic arm - 2-finger Bio Gripper - 6-axis Force / Torque sensor |
| --- | --- |

BEFORE YOU START

The system starts automatically. During the first few seconds:

- Release the handleThe Inverse3 handle will gently pull itself to its center position —

- Wait for the “Starting control...” message before picking up the handle

- Important: The rotating base of the Inverse3 must be set to its

center/zero position manually before each session.

STYLUS CONTROLS

| Button | Action | Description |
| --- | --- | --- |
| Move handle | Move robot | The robot arm mirrors your hand movements in real time |
| Button A | Gripper open / close | Press to close the gripper, release to open it |
| Button B | Wrist rotation ON/OFF | First press: enables wrist control from stylus rotation. Second press: arm slowly returns to gripper-down position |
| Squeeze grip | Haptic feel | The handle pushes back when the robot contacts an object |

SAFETY GUIDELINES

⚠️ Move Slowly The robot mimics your movements at scale. Sudden or fast movements of the Inverse3 will cause the robot to lurch. Move the handle slowly and deliberately, especially near objects.

⛔ Avoid High-Speed Collisions Do not drive the robot into the table surface or obstacles at speed. Approach surfaces slowly. The haptic feedback will push back when contact is detected — use this as your cue to stop.

ℹ️ Haptic Feedback The force-torque sensor at the robot’s wrist measures contact forces and plays them back through the handle. Light touches produce gentle resistance; hard contacts produce stronger pushback. If the handle feels like it is pulling you toward something, stop moving.

Questions? Ask a team member at the booth.